GAS HEAT PUMP (GHP)
FREQUENTLY ASKED QUESTIONS (FAQs)

• WHAT IS A GAS HEAT PUMP (GHP)?

GHPs move heat from an external heat source to a heat sink (indoors) using natural gas as the primary energy source to drive the thermodynamic cycle. Heat pumps extract heat from air, water or the ground, and then move (pump) this heat to the inside space. Heat pumps are very efficient, exceeding the 100% efficiency barrier, something conventional natural gas HVAC equipment cannot achieve.

• HOW DO GHPs WORK?

Heat pumps in heating mode are effectively air conditioners running in reverse, extracting heat from outdoors and efficiently moving it indoors. Engine-driven GHPs work very similar to electric heat pumps, with the electric motor replaced by a gas engine. Absorption GHPs replace the motor and compressor with a generator and absorber running on gas.

• WHAT ARE THE BENEFITS OF A GHP?

GHPs lower heating and may lower cooling costs, reduce Greenhouse Gas Emissions (GHG) and provide a reliable system that does not require a back-up heating source.

• ARE GHPs ENERGY EFFICIENT?

Yes. For heating, GHPs offer Coefficient of Performance (COP) ratings between 1.2 to 1.4 or greater. For cooling, COP is typically 1.3 for an engine-driven GHP or 0.6 for absorption. Even at 0.6, absorption GHPs still tend to save money over conventional cooling systems. When simultaneously cooling and heating, COPs can reach as high as 3.0.

• HOW WILL A GHP IMPACT MY GAS BILL?

GHPs reach up to 140% heating efficiency compared to 80% for an atmospheric gas furnace or boiler, which leads to savings of 30-50%. When using a GHP for cooling, gas bills increase during cooling season, but electric bills may be lower.

• DO GHPs STILL USE ELECTRICITY?

Yes. Like a gas furnace, GHPs use electricity for controls, fans, pumps, etc. Electric usage is generally small and does not require costly electric upgrades.

• CAN I REPLACE MY EXISTING ELECTRIC HEAT PUMP WITH A GHP?

Absolutely. When natural gas is available a GHP saves money, provides warm comfortable heat and does not require a backup/emergency heat system like electric heat pumps do in colder climates.

• DO GHPs OPERATE BELOW 350F (1.70C)?

Yes. Heat recovered from the combustion process allows GHPs to operate below 00F (-17.80C) without the need for back-up heating. In comparison, standard electric heat pumps typically switch over to electric resistance or emergency heat mode when the outside temperature drops below 350F (1.70C).

• WHAT IS THE AVERAGE LIFESPAN OF A GHP?

GHPs typically have a lifespan of 20 years.

• WHAT MARKETS & APPLICATIONS ARE BEST FOR GHPs?

GHPs work well in commercial and residential space and ventilation heating, DHW heating and space cooling. GHPs are already available for commercial markets.

• WHO MAKES GHPs?

Search manufacturers at https://gasairconditioning.com/technologies/heat-pumps/resources/

• DO GAS HEAT PUMPS IMPROVE THE ENERGY EFFICIENCY CLASS OF BUILDINGS?

Yes, the energy efficiency class of a building is affected by total energy consumption which is lower with GHPs.

• CAN GHPs OPERATE ON RENEWABLE GAS?

Yes. GHPs can run on renewable gas (RNG) and blends of hydrogen for significant greenhouse gas reductions.

WHAT ARE THE MAIN DIFFERENCES BETWEEN THE INSTALLATION OF A GAS HEAT PUMP VERSUS A BOILER?

There are no differences, the gas supply and hot water distribution systems are similar for both systems.

• WILL A BLACKOUT AFFECT THE OPERATION OF A GAS HEAT PUMP?

The primary energy source is natural gas.  A back-up generator can be sized about 75% smaller than for electric cooling.

For more information on GHPs such as sizing, availability, and case studies,

Please visit www.gasheatpumps.com.